TECHNICAL RIDER

INTARS BUSULIS + BAND 7

SOUND

GENERAL INFORMATION:

In case of problems meeting this specification, please do not hesitate to contact the artist through your contact. We ask local promoters or sound engineers not to make changes in this spec, on behalf of the artist, without confirming with artist or representative. Brands and types of sound equipment is meant as quality guidance, and not as absolute requirements if not stated specifically.

EQUIPMENT QUALITY:

Sound equipment must be of high standard and free from audible noise. Mixing desk must be in a good position in the audience area, centred between left and right PA to ensure good control possibilities. Sound system must be rigged and tested for functionality before artist arrives for setup and sound check. Local system engineer must be available for the artists own engineer at all times from sound check to end of concert.

SOUND SYSTEM:

High quality 3 or 4 way active PA-system. The system must cover all audience areas with uniform and undistorted sound. It is important that the system is free of hum and noise. (pref: L’Acoustics, d&B audiotechnik, Meyer Sound, or similar).

FOH:

Only Yamaha CL series or TF Series!!! We have a working show file on them, and we are running 6ch playback from DANTE NETWORK!!!!

No Yamaha LS9, Soundcraft, Behringer, Allen&Heath or wonders  like that!

In case that You can not offer desk, we will travel with our own. Then please provide 24 channel splitter on stage (if we are not the only band)  and CAT5 between FOH and stage. Cost of our desk – 150 EUR!!!

MONITOR SYSTEM:

4 x wedges min 350w on 3 separate mix from FOH or monitor mix.

4 x w/less IN EAR mono systems (Sennheiser ew300 G3, Shure PSM) on 4 separate mix from FOH or monitor mix.

2 x XLR on 2 seperate mix from stage box to drummers IN EAR MIXER (You should provide!!! Any 4ch mixer with working headphone output)

2 x wedges on stands as side fills if stage is wider than 10 meters

MICROPHONES/STANDS:

See channel list. All microphones can have alternates, just please contact me!

BACKLINE

DRUMS:

Fully professional drum set in working order (TAMA, GRETSCH, YAMAHA, SONOR, DW)

Bass drum – 20’’-22’’

Bass drum pedal

Snare drum with stand 14’’-15’’

Hi Hat stand

Rack tom – 10’’

Rack tom – 12’’

Floor tom – 14’’

4 cymbal stands

Drum throne

Cymbals – 2 crash, 1 ride, Hi Hat, 1 splash, (Zildjian K, A or Avedis, Sabian HHX or A

2 x SCHUCO 220V power plugs EU

3m x 2m drum riser with carpet

Plexiglass DRUM SHIELD (we have 8 vocals on stage & loud drummer)!!!

GTR + Acc GTR:

Fender Hot Road de Ville, Twin Reverb or similar guitar combo

2 x Accoustic guitar stands!!!!!

2 x SCHUCO 220V power plugs

BASS:

Eden Traveler WT550 with 4 x 10’’ cabinet (or similar according to the venue  NO AMPEG SVT-3)

1  x guitar stand

2 x SCHUCO 220V power plugs EU

KEYBOARD:

1 x Yamaha CP33 or Yamaha Motif ES8, EF8, XS8 digital stage piano with 88 HARD keys

1 x „x” type strong Keyboard stand

1 x  piano bench or similar

2 x schuco 220v plugs EU

CONTACT INFORMATION:

GATIS BALUTIS

SOUND ENGINEER

GSM: +371 28837727

gatis.balutis@gmail.com

INTARS BUSULIS + BAND 7

CHANNEL  LIST

| Ch | Description | Mic/d.i. | Stand |
| --- | --- | --- | --- |
| 1 | BD | BETA 52 or similar | LOW |
| 2 | SN TOP | SM 57 or similar | LOW |
| 3 | SN BOT | BETA 57 or similar | LOW |
| 4 | HH | SM 81 or similar | LOW |
| 5 | T1 | E904 or similar | CLAMP |
| 6 | T2 | E904 or similar | CLAMP |
| 7 | T3 | E904 or similar | CLAMP |
| 8 | OH L | KSM 32 or similar | BOOM |
| 9 | OH R | KSM 32 or similar | BOOM |
| 10 | BASS | line from combo |  |
| 11 | GTR | e906 or similar | LOW |
| 12 | Acc GTR | XLR from pedal board |  |
| 13 | Key L | d.i.box |  |
| 14 | Key R | d.i.box |  |
| 15 | SAX Tenor | Beta 98 |  |
| 16 | SAX Soprano | RE20 or similar | LOW |
| 17 | Voc Key | Beta 58/SM58 | BOOM |
| 18 | Voc GTR | Beta 58/SM58 | BOOM |
| 19 | Voc SAX | Beta 58/SM58 | BOOM |
| 20 | Voc Bass | Beta 58/SM58 | BOOM |
| 21 | Voc Drums | Beta 56 or similar | BOOM |
| 22 | Voc Lead Busulis | Neumann KMS-105 (only) | STRIGHT |
| 23 | Back Voc 1 | Beta 58/SM58 | BOOM |
| 24 | Back Voc 2 | Beta 58/SM58 | BOOM |
| 25 | Playback 1 | DANTE | on FOH |
| 26 | Playback 2 | DANTE | on FOH |
| 27 | Playback 3 | DANTE | on FOH |
| 28 | Playback 4 | DANTE | on FOH |
| 29 | Playback 5 | DANTE | on FOH |
| 30 | Playback 6 | DANTE | on FOH |
|  | AUX SENDS |  |  |
| Aux 1 | VOCAL LINE FRONT | 2 x wedges on floor |  |
| Aux 2 | KEYBOARD | 1 x wedge on floor |  |
| Aux 3 | GTR | 1 x w/less in ear system |  |
| Aux 4 | SAX | 1 x w/less in ear system |  |
| Aux 5 | BASS | 1 x wedge on floor |  |
| Aux 6 | BACK VOCAL 1 | 1 x w/less in ear system |  |
| Aux 7 | BACK VOCAL 2 | 1 x w/less in ear system |  |
| Aux 8 | Drums MIX | xlr to drummers mixer |  |
| Aux 9 | Drums CLICK | xlr to drummers mixer |  |

PLEASE SEND ME AN FULLY PROVIDED SOUND EQUIPMENT SPECIFICATION AT LEAST ONE WEEK BEFORE SHOW!

gatis.balutis@gmail.com GSM: +371 28837727